# REII Open Questions and Decisions

Oct 29, 2026 · @Adriaan

These are all the questions still open for REII across the four packages, plus five decisions only Adriaan can make. They come from the Increment 4 README, the progress file and the ICD's magnet section (IF12.0). Tick each item off as its answer comes in.

## EEII 2 (magnet)

- ☐ Current sensor on J6.8: which part it is, its supply voltage, its volts per amp and its output at 0 A. The output must stay at or below 3.3 V at the full 7.5 A. A sensor on a 5 V supply that idles at 2.5 V could go past 3.3 V.

- ☐ Holding threshold: the current that counts as "holding". The crane reports holding when the current reaches the threshold within 2 s.

- ☐ MAG_LOGIC on J3.6: the ICD says "Output / Input… enable / status line". Is it an enable from the crane, a status signal back from their board, or unused? It is currently set as an output that is HIGH while the magnet is on.

- ☐ PWM frequency: the magnet PWM is set to 20 kHz, the ICD maximum. Can their MOSFET module and logic level converter switch that fast, or do they want it lower?

- ☐ Hardware ready: are the switch module and the 3.3 V to 5 V converter built, and can they bring them for the 31 Oct test and the 6 Nov rail session?

- ☐ Release: does the load drop cleanly when the PWM goes to 0, or is there residual magnetism?

- ☐ Emergency stop: is the 24 V magnet supply cut by the red E-stop? If so, pressing it drops the load. The crane's lost-link response keeps the magnet on, so this is the one case where the load falls.

## MCTR 2 (hoist and load cell)

- ☐ Spool and gearbox: spool radius and the total gear ratio (all six stages plus the worm).

- ☐ Brake on J4.8: is it wired, does HIGH release it, does the 50 ms release time suit them, and what powers it?

- ☐ Load-cell capacity: what is the rated capacity on the label? The default scale assumes 20 kg. Calibrating with the known masses replaces this anyway, but a 50 kg cell would read 2.5 times too low until then.

- ☐ Known masses: two or three for the 6 Nov rail session.

- ☐ Hoist motor nameplate: poles and rated speed at 50 Hz (or ask EEII 1). The hoist position is estimated from drive frequency, gear ratio and spool radius, so it needs these.

- ☐ Hoist stroke: confirm 1.6 m.

## EEII 1 (VFD)

These are already in the drafted follow-up message to EEII 1.

- ☐ Current sensor: is the output a 10 A trip flag, PWM or a serial bus? Which pin, and what scale if it sends a value?

- ☐ DRV_FAULT on J3.5: does their board have a fault output, and is a fault HIGH or LOW?

- ☐ Drive limits: start frequency, maximum frequency, V/f limit and dead time.

- ☐ Joint hook-up: can we book 12 Nov?

## MCTR 1 (trolley)

- ☐ Encoder output: push-pull or open collector? This decides between the 2.2 kΩ / 3.9 kΩ divider and a 4.7 kΩ pull-up to 3.3 V.

- ☐ Motor test: their result from Monday 2 Nov.

- ☐ Gear ratio: a quick confirmation, if they can. Otherwise the C1 test settles it by counting 10 wheel turns.

- ☐ Rail session: confirm 5 Nov (proposed in the reply).

- ☐ Rail length: exact value, measured on 5 Nov.

## Group

- ☐ Schedule: rail slots on 5, 6 and 9 Nov, the demo order and the presentation template.

- ☐ ICD Rev B changes: agreement to the VFD current signal moving off J6.7, any gear-ratio change, and the E-stop input if Adriaan goes ahead (decision 1 below).

## Decisions for Adriaan

- E-stop input to the crane MCU. The ICD gives the MCU no E-stop input, so the "E-stop" lamp on the dashboard can never light.

  - Propose a spare contact on the E-stop wired to J3.7 (free) as an ICD Rev B change. The E-stop owner has to fit it.

  - Or leave the E-stop as HV-only and say so in the report.

- Pin for EEII 1's current signal, once they say what it is.

  - J7.11: it was the second encoder channel, now unused. It is a direct pin that can count pulses, so it suits a PWM or pulse output.

  - J3.8 (or J3.7, if not used for the E-stop): fine for a plain trip flag.

  - J7.10: free since the CE line moved to J3.3.

- Magnet holding threshold if EEII 2 doesn't answer by 31 Oct. Suggested: 6.0 A, 80 % of 7.5 A.

- Hoist fallback for 13 Nov. If EEII 1's board isn't signed off by 12 Nov, show the hoist as brake plus drive signals on the scope, with hoist position still simulated? The current plan assumes yes.

- Gear ratio if MCTR 1 can't confirm. No decision needed: use the count from the C1 test.